Chapter 1

1.1 Research Background

The massive evolution of Internet of Things (IoT) has transformed significantly the modern infrastructure with embedding connectivity into sectors from industrial automation and healthcare, smart home environments to consumer electronics, expanding the demand for efficient, low-latency communications, which has driven the widespread adoption of lightweight messaging protocols. The Message Queuing Telemetry Transport (MQTT) protocol is the most used one among others which has emerged as a de facto standard for machine-to-machine (M2M) communication. This protocol is designed primarily for bandwidth-constrained environments such as oil pipeline monitoring over satellite links because MQTT’s “publish-subscribe” architecture minimizes network overhead and code footprint. This efficiency makes MQTT suitable for resource-constrained devices and environments, which often operate with limited processing power and memory.

However, the good characteristics of MQTT such as efficiency, simplicity, and openness, often introduce significant security vulnerabilities when deployed in untrusted environments because MQTT implementations frequently lack robust security controls by default. Also, standard brokers often operate without encryption such as TLS or authentication, relying on plain text communication which can lead to data interception, tampering and unauthorized access. Furthermore, if the Access Control Lists (ACLs) are not strictly enforced, the protocol’s topic-based structure allows for predictable targeting even though that structure is flexible, creating a fragile security posture in which a single node compromise or the interception of traffic can threaten the integrity of entire network or infrastructure.

While protocol-level vulnerabilities in MQTT are well-documented and frequently analyzed in isolation from the physical environments in which this MQTT protocol and devices interact and operate in most of the academic literature. The deployment of IoT devices is often in physically not so secure environments, such as lobbies, outdoor sensors, or shared office spaces, which can be easily accessible to malicious actors, opening a critical attack vector which is “Human Interface Device (HID)” attacks. Programmable microcontrollers such as Raspberry Pi Pico can be configured to emulate trusted USB peripherals like keyboards, and these USB can be tagged as “BadUSB” which can inject malicious payloads into a host system in very short amount of time, such as milliseconds, bypassing traditional network firewalls and air gaps by leveraging the inherent trust operations systems place in a human input devices.

Current research focuses mostly on either securing the MQTT protocol via encryption and authentication or analyzing physical security as a separate domain, pointing out a distinct lack of comprehensive studies that investigate the convergence of these two threat vectors. In real-world scenario, an attacker is unlikely to restrict themselves to a single vector, after that they will exploit the path of least resistance, often using physical access to establish a foothold for a network-level attack. This gap is addressed by this thesis by exploring a chained attack methodology, utilizing a physical HID injection to compromise a gateway or host, facilitating a subsequent protocol-level exploitation of an MQTT network. By simulating this multi-vector threat using accessible hardware, specifically the Raspberry Pi Pico and ESP8266, aiming to demonstrate the fragility of current IoT deployments and evaluate the effectiveness of practical, lightweight countermeasures.

1.2 Problem Statement

As of now, the security methodologies applied to the IoT ecosystems remain critically fragmented, pointing out a fundamental disconnect between the analysis of protocol-level threats. MQTT security research predominantly focuses on network-layer exploits such as Man-in-the-Middle (MitM) attacks, Denial of Service (DoS), and lack of encryption (TLS) in recent studies, often assuming the attacker is already on the network or that the network boundary is the primary line of defense. Conversely, research into Human Interface Device (HID) attacks, typically utilizing tools like the “USB Rubber Ducky” or modified microcontrollers, primarily focuses on local privilege escalation or data exfiltration from a specific workstation.

This gap or separation creates a dangerous blind spot, because as I mentioned earlier, IoT deployments are mostly in easily assessable areas physically and MQTT is insecure by default, or the USB ports are left unguarded. However, the intersection of these two weaknesses is rarely tested or understood as a unified or chained threat model.

This thesis addresses this specific problem of the vulnerability of MQTT-based IoT environments to chained, multi-vector attacks where a low-resource physical intrusion serves as the gateway for a high-impact network compromise. Security professionals lack a unified, low-cost framework for this specific attack chain simulation. Without a reproducible methodology to demonstrate how a rogue HID device like a Raspberry Pi Pico can programmatically inject scripts to exploit a MQTT broker, organizations or enterprises remains unaware of how easily their so called “secure” air-gapped or firewalled networks can be breached via simple physical access.

The defense mechanisms for such scenarios are underdeveloped for constrained hardware. Standard Intrusion Detection Systems (IDS) are often too resource-heavy for microcontrollers like the Arduino UNO or ESP8266, and thus consequently, there is a critical need for a lightweight, anomaly-based detection framework capable of identifying the specific traffic patterns generated by these hybrid attacks. Until such a framework is designed and evaluated, IoT networks remain susceptible to cascading failures triggered by a single moment of physical negligence.

1.3 Research Questions

This research aims to answer the following specific questions:

1) What are the specific vulnerabilities in MQTT implementations which can be exploited on constrained IoT hardware such as ESP8266?

The focus of this question is to identify protocol-level weaknesses that are feasible to exploit on low-power devices covering issues such as weak authentication, unencrypted traffic, topic hijacking, and Quality of Service (QoS) abuse. The outcome will provide guidance about the selection of realistic attack scenarios for the testbed.

2) How can a physical-level access attack using Raspberry Pi Pico in HID-based mode in combination with MQTT protocol exploitation to create chained multi-vector attack?

The objective of this question is to focus on developing and demonstrating a coordinated attack pattern or flow which starts with physical access (via USB HID emulation in this case) and transitions into network-level MQTT exploitation, exploring how these two attack vectors can be integrated to bypass layered security measures.

3) What countermeasures can be implemented to detect or mitigate these combined physical and network-based attacks without significantly impacting device performance?

This question is about investigating the practicality of deploying enhanced security controls such as TLS (Transport Layer Security) or ACLs (Access Control Lists), IDS (Intrusion Detection System) or anomaly detection on constrained devices aiming to evaluate trade-offs between security and resource consumption.

4) How can a low-cost, reproducible, and safe testbed be designed to simulate combined protocol and hardware exploitation for academic and training purposes?

This question aims to the development of a replicable research and teaching platform which can be adopted by cybersecurity educators, penetration testers and IoT developers ensuring this testbed remains safe and secure for controlled experiments.

1.4 Research Aim and Objectives

Aim: The primary aim of this research is to design, implement, and evaluate a novel, hybrid attack framework that integrates physical Human Interface Device (HID) exploitation with network-level MQTT protocol abuse. By utilizing low-cost, accessible hardware, specifically the Raspberry Pi Pico as an attack vector and the ESP8266 as a target, this study aims to demonstrate how physical access can be "chained" to compromise an isolated IoT network. Furthermore, the research seeks to develop and validate a lightweight, anomaly-based Intrusion Detection System (IDS) capable of identifying these multi-vector threats on resource-constrained microcontrollers.

Objectives:

1) In-Depth Literature Review – To conduct a comprehensive literature review on MQTT vulnerabilities, IoT device security challenges, and HID-based physical attacks to establish theoretical foundation and identify research gaps.

2) Development of Modular IoT Attack Testbed – To develop a modular IoT attack testbed integrating an MQTT broker, ESP8266 clients, and a Raspberry Pi Pico for controlled protocol-level and hardware-level attack simulations with an establishment of a wired monitoring node (Arduino UNO) with Ethernet Shield.

3) Design and Implement a Lightweight Intrusion Detection System (IDS) – This is done on the Arduino platform, and the objective is to create an anomaly detection logic which can identify unauthorized publish events or suspicious traffic pattern in real time and trigger immediate physical alerts to a small LCD display connected.

4) Evaluation on the Effectiveness of Proposed Countermeasures – To demonstrate practical countermeasures with TLS encryption, Access Control Lists (ACLs), and payload validation to mitigate combined exploitation attacks while maintaining device performance determining the most effective mitigation strategies for constrained IoT environments.

1.5 Scope and Limitations

Scope: This scope of this research is strictly defined to ensure technical depth and feasibility within the project timeframe.

- Protocol Focus: The research focuses exclusively on the MQTT protocol (specifically versions 3.1.1 and 5.0 as supported by Mosquitto). Other IoT protocols such as Zigbee, CoAP or LoRaWAN are excluded unless they interact directly with the MQTT gateway.

- Hardware Selection: This experimental testbed is standardized on specific, widely available microcontrollers, the Raspberry Pi Pico (RP2040) for HID emulation, the ESP8266 (NodeMCU) for Wi-Fi connectivity, and the Arduino UNO (ATmega328P) with an Ethernet Shield (W5100) for networking monitoring or IDS.

- Attack Vector: The attack simulations are limited to HID-based physical injections which in other words is BadUSB leading to protocol abuse. Other physical abuse methods such as chip desoldering, power analysis or JTAG debugging are outside of the scope.

- Environment: All experiments are conducted in a strictly isolated, offline sandboxed laboratory network to prevent any accidental interference with public networks or production systems.

Limitations: The following constraints are included in this research’s operation.

- Resource Constraints: The IDS is implemented on the Arduino UNO, which has limited processing power of 16Mhz and a memory of 2KB SRAM. The detection logic is therefore limited to “Signature-based” or “Anomaly-based” rules, however, complex Machine Learning (ML) models cannot be supported which are often found in server-side IDS.

- Wired vs Wireless IDS: The IDS implementation utilizes a wired Ethernet connection for reliability and to distinguish it from the wireless target. Thus, the specific vulnerabilities of Wi-Fi jamming or de-authentication attacks will not be evaluated.

- Host Dependency: The HID attack vector requires the presence of a host machine such as a pc, laptop or a gateway with an exposed USB port and an unlocked session. Environments where physical USB ports are physical blocked or disabled via Group Policy Objects (GPO) are not accounted.

1.6 Significance of the Study

This thesis holds significant value in three distinct domains: The Academic Body of Knowledge, Practical Cybersecurity Operations, and Broader Societal Safety, bridging the gap between physical and network security, and providing tangible contributions to understanding the mitigating IoT threats.

Academic Significance: The primary academic contribution of this thesis is the development of novel hybrid attack model which integrates physical Human Interface Device (HID) injection with a network-level MQTT protocol exploitation. While existing literature extensively covers MQTT vulnerabilities such as lack of encryption or weak authentication and separate studies address HID attacks, there is scarcity of research that empirically demonstrates the convergence of these two vectors, and thus this study provides a reproducible framework and methodology for researchers to simulate multi-stage attacks, offering empirical data on how “low-tech” physical access can be weaponized to bypass “high-tech” network defenses. Furthermore, the development of a lightweight Intrusion Detection System (IDS) on constrained hardware, in this case is the Arduino UNO, challenges the assumption that effective anomaly detection requires high-performance computing resources, contributing new proof to the field of embedded systems security.

Practical Significance: This research offers a verifiable, low-cost reproducible testbed which can be utilized for red-teaming and vulnerability assessment training which is valuable for the cybersecurity industry. The methodology documented here can be adopted by the security professionals and penetration testers for the simulation of insider threats or physical breaches in environments such as Industrial Control Systems (ICS), smart homes and smart offices. Additionally, this study highlights the critical dangers of default configurations which are useful for IoT hobbyists and researchers. Demonstration of unauthenticated broker compromise using the script injection shows actionable insights into the necessity of hardening firmware and enforcing strict Access Control Lists (ACLs) especially during the development phase rather than the post-deployment patch.

Societal Significance: As IoT becomes increasingly ubiquitous in critical infrastructure from Healthcare to Education and Household, the integrity of MQTT protocol becomes a matter of public safety in an aspect. The fragility of the interconnected IoT systems is pointed out in this study and consequently raises the awareness among the stakeholders. By demonstrating how a microcontroller like Raspberry Pi Pico can disrupt a network, the study proves its defense-in-depth approach, ultimately contributing to more resilient infrastructure that protects user privacy and prevents service disruptions in essential daily applications.

1.7 Thesis Organization

This thesis is organized into six chapters, systematically guiding the reader from the theoretical foundation to the practical foundation demonstration and analysis of the research findings.

- Chapter 1: Introduction – This chapter outlines the research background and problem statement of the security testing, establishing the aim, objectives, and scope of the study, and highlighting the significance of investigating chained physical and protocol attacks in IoT environments.

- Chapter 2: Literature Review – This chapter provides a critical analysis of recent existing literatures and academic works related MQTT security, IoT architecture, and HID-based attack vectors, synthesizing current knowledge to identify the specific research gap which is the lack of integrate multi-vector attack simulations, which this thesis addresses.

- Chapter 3: Methodology – This chapter is about the details of the technical design and implementation of the experimental testbed, documenting the hardware configurations with Raspberry Pi Pico, ESP8266 and Arduino UNO together with the ethernet shield and the network topology of the isolated sandbox, and the development of the software payloads which is used for both the attack and the intrusion detection system (IDS).

- Chapter 4: Results – This chapter presents the empirical data gathered from the experiments, including logs, visual evidence of the baseline attack success rates, the performance latency of the attack chain, and the successful triggering of the Arduino-based IDS alerts.

- Chapter 5: Discussion – This chapter is about interpreting the findings from the previous chapter, which is Chapter 4, analyzing the effectiveness of the attack vectors and the responsiveness of the detection mechanisms, and evaluating the impact of the implemented countermeasures such as Access Control Lists (ACLs), and discusses the trade-offs between security and performance on constrained devices.

- Chapter 6: Conclusion – This chapter is simply about summary of the key research findings, and discusses the limitations of the study, and offers recommendations for future research directions and enhancements for a better security of MQTT-based IoT ecosystems.