Chapter 2

Abstract

The Message Queuing Telemetry Transport (MQTT) is a protocol which has become foundational to the IoT system because of its lightweight nature. It is a publish-subscribe protocol and together with its simplicity, inherent security risks come to play such as lack of proper authentication. In some cases, secure environments used in combination with insecure messaging protocols are often attacked by physical attack vectors such as USB payload injections often known as Bad USB. In this literature review, current research across five key areas is synthesized in which MQTT security vulnerabilities, IoT exploitations, USD HID attacks using Raspberry Pi Pico and existing offensive security toolkits. This review finds that although extensive MQTT-related studies exist, there is a clear fact that a gap is present in combination with protocol and physical-layer exploitation using embedded hardware such as microcontrollers. To justify that gap, this proposed thesis research aims to build a framework which can be reproducible for practical applications of Offensive Security testing of MQTT-enabled IoT ecosystems using the microcontrollers Raspberry Pi Pico and ESP8266.

Keywords: MQTT, IoT, USB HID, ESP8266, Raspberry Pi Pico, Offensive Security

# 2.1 Introduction

Modern lifestyle has been transformed by the Internet of Things (IoT) from daily gadgets such as smartwatches and smart homes to industrial automation. Efficient communication protocols play an important role for the efficient communication of these systems for handling high volumes of data from different devices as IoT is a diverse realm. A lightweight publish-subscribe protocol named MQTT becomes a preferred standard communication protocol due to its minimal overhead and simplicity. However, MQTT lacks strong security and its deployment in critical infrastructure often lacks the required level of security such as authentication, encryption or message integrity verification.

This weakness poses significant hazards, especially when using MQTT in unprotected situations or when attackers obtain physical access to network-connected hardware. The increasing availability of low-cost microcontrollers such as the Raspberry Pi Pico and ESP8266 has allowed offensive teams to simulate or spoof trusted devices, exploit open MQTT brokers, and even inject commands via USB-based payloads. While this research has focused on MQTT vulnerabilities and USB HID attacks separately, there is scant evidence of a comprehensive approach that addresses both vectors evaluating existing research in MQTT protocol security exploitations with IoT vulnerabilities, USB HID exploitations and offensive security toolkits.

# 2.2 Overview of MQTT in IoT Environments and its Security Considerations

MQTT is a lightweight and open messaging protocol primarily designed for constrained environments and unreliable networks operating on a publish-subscribe model where clients communicate through a broker which handles topic-based message delivery. Its efficiency and simplicity make it ideal for IoT applications in which home automation, industrial telemetry and even medical devices are included. MQTT is developed by IBM in 1999 and later standardized by OASIS, enables machine-to-machine (M2M) communication via a central broker that physically and temporally separates publishers (data senders) from subscribers (data receivers).

MQTT operates over TCP/IP and three Quality of Service (QoS) levels are defined: “QoS 0” means “at most once”, “QoS 1” means “at least once” and “QoS 2” means "exactly one” (HiveMQ Team, n.d.). Message reliability is contributed by QoS levels and on the other hand, higher QoS levels demand more resources leading to latency and network traffic which is good for resource-constrained IoT devices(Joseph Keenan & Joseph, 2025).

Although the presence of the mentioned features, the default security posture of MQTT is minimal as I mentioned earlier. For example, in case with the implantation of authentication is needed, the username and password are in plain text model and hence making it susceptible for sniffing and brute-force attacks (Kurdi & Thayananthan, 2021,Laghari et al., 2024, ŞİMŞEK & ATILGAN, 2023). Martin et al. (2022) warn that unencrypted MQTT traffic can leak sensor data, user credentials, and device states to passive attackers. MQTT is popular because of its low power and bandwidth needs. However, these same characteristics make the protocol vulnerable because it does not enforce security measures like encryption, mutual authentication, or integrity checking. Furthermore, many implementations leave MQTT brokers with default settings like open ports and anonymous access. Even though TLS/SSL encryption can be added, additional computational overhead is met and thus making it unsuitable for resource-constrained devices (Kurdi & Thayananthan, 2021, Laghari et al., 2024).

It is clearly indicated that there is a lack of intrinsic security in MQTT’s design and consequently require additional external countermeasures and careful implementation for safeguarding the secure IoT deployments. Despite these vulnerabilities, MQTT remains widely implemented with few security enhancements, making it an enticing target for attackers. There is a lighter variant of MQTT protocol which operates over UDP, which is named MQTT – SN: MQTT for Sensor Networks primarily designed for even more constrained environments. Even though it simplifies the protocol, but different security challenges could be introduced because of the connectionless nature of UDP protocol (Roldán-Gómez et al., 2022).

A huge number of MQTT brokers on the public internet are set up with poor or no authentication. Using honeypots, they determined that more than 60% of brokers enabled anonymous access, which exposed critical information to any connected client. This scenario is aggravated by out-of-date documentation and low developer awareness of acceptable practices (Alghamdi et al. 2021). Topic Hijacking and Message Spoofing are also major security concerns; Topic Hijacking occurs when an attacker subscribes to or publishes data on a topic which is reserved for critical devices. It is demonstrated by El-Khatib (2019), that by using the MQTT wildcard characters, attackers can eavesdrop on all subtopics, (Example: “Sensors/#”) or can overwrite the actuator commands. In smart home or industrial systems, message spoofing is extremely dangerous in which harmful automation actions could be resulted by the forged data.

Fuzzing-based tests on MQTT brokers were performed to detect erroneous packet formats that could cause broker crashes. Denial-of-service attacks were shown using enormous numbers of retained messages and persistent subscriptions. Although rate-limiting and client authentication can help counter these attacks, they are rarely implemented in default broker configurations (Zhao et al. 2022).

# 2.3 Role of Raspberry Pi Pico and ESP8266 in IoT Security Research

The choice of hardware is crucial for research into IoT security for practical ground, particularly protocol exploitation. The chosen devices must be programmable, capable of network communication, and compatible with the resource-constrained nature of multiple IoT endpoints. The Raspberry Pi series such as Raspberry Pi 3 models or Raspberry Pi 4 models are widely adopted in academic testbeds because of their versatility, processing power and connectivity options (Alatram et al., 2023; D'Ortona et al., 2022; Hussein & Nhlabatsi, 2022; Karn, 2024; Polisiani et al., 2025; Vaccari et al., 2021). These devices often serve as MQTT brokers, publishers or subscribers and even the most critical part is that they can perform as attacker tools due to their ability to run custom scripts or various penetration testing tools (Alatram et al., 2023; Alsabbagh et al., 2024; ŞİMŞEK & ATILGAN, 2023). For instance, in the Alatram et al. (2023) paper utilized a number of sixteen Raspberry Pi 3 Model B+ devices as publishers and subscribers in a physical IoT testbed for the DoS/DDoS MQTT traffic generation. Similarly, in 2022 (Hussein and Nhlabatsi), Raspberry Pi 3 B+ was employed as a source of attacks against on IKEA TRÅDFRI lighting system which utilized to host a ZigBee2MQTT library and for exploitation, CoAP is used.

ESP32 microcontrollers and ESP8266 (e.g., ESP8266 NodeMCU, ESP8266 D1 Mini V2) on the other hand, are also prominent in representing the constrained IoT devices which are often targets or unwitting participants or evidence in attacks. They got integrated Wi-Fi and in ESP32, it also got Bluetooth Low Energy (BLE), also they can be programmable with Arduino IDE making them ideal or custom firmware deployment and interacting with MQTT brokers (Keenan, 2025; Kodali & Mahesh, 2016; D'Ortona et al., 2022). It is clearly demonstrated that a low-cost MQTT implementation using ESP8266 interfacing with sensors can act as an IoT client efficiently (Kodali and Mahesh, 2016). (Keenan, 2025) tested a variety of encryption algorithms on ESP8266 D1 Mini V2 devices for assessing their performance on resource-constrained platforms. And ESP8266 NodeMCU is directly employed as a sensor node demonstrating practical MQTT-based Man in the Middle Attack (MitM) and DoS and brute-force attacks capturing network traffic over Wi-Fi (ŞİMŞEK & ATILGAN, 2023) which will be discussing in detail in the next section. Delgado et al. (2023) also demonstrate remote cybersecurity labs that use Arduino boards, some with Wi-Fi capabilities comparable to ESP devices, and a Raspberry Pi server, demonstrating how these combinations allow for actual cybersecurity exercises.

The Raspberry Pi Pico is an addition to Raspberry Pi family, which was released in 2021, with its RP2040 microcontroller, it is tagged as a lower-cost, smaller and power-efficient option. While not as commonly cited in the literature for complicated network-level assaults as its more powerful cousins, its properties make it ideal for portraying tightly limited edge devices in an IoT network. Its distinct blend of simplicity and performance for its size suggests that it could be effectively used in demonstrations where a small footprint or direct hardware interaction is critical, such as "Bad USB" assaults replicated in Karn (2024), in which a Raspberry Pi Pico served as a bad keyboard. Shah and Mathews (2022) investigated the use of programmed HID attacks on the Raspberry Pi Pico and related devices such as Digispark and Teensy. Their research showed that attackers may use keyboard injections to distribute scripts that exploit command-line interfaces, download payloads from distant servers, and change local files. However, their research concentrated on Windows-based privilege escalation and did not look into how USB-based payloads may be used to launch attacks on network-connected devices like MQTT clients.

Raspberry Pi Pico's flexibility allows it to act as an initial access point in a multi-stage attack. For example, the Pico might be configured to send a PowerShell or Python script to a host computer, which connects to a MQTT broker and launches a predetermined assault. This includes subscribing to critical topics, flooding the broker with bogus messages, and activating ESP8266 devices that are programmed to respond to certain payloads. Despite the potential of this route, scholarly literature has yet to show a completely integrated attack chain incorporating HID payload injection and MQTT protocol exploitation. Furthermore, because the Pico is not inherently suspicious (it is frequently considered as an instructional gadget), it may go undetected in workplaces with weak USB device restrictions. The thesis expands on this underutilized use case by including Pico into a physical attack scenario in which USB-based execution results in MQTT-based network abuse.

# 2.4 Protocol Exploitations on MQTT-Based IoT Devices

As I mentioned earlier, MQTT's design philosophy of being lightweight and simple, while beneficial for IoT, exposes risks if not properly secured. The following will identify numerous types of exploitation attempts that target MQTT, frequently utilizing default setups or architectural peculiarities.

## 2.4.1 Denial of Service (DoS/DDoS) Attacks

One of the most prevalent threats in IoT ecosystems is DoS and DDoS attacks which aim to overwhelm target systems or services with massive amount of traffic to disrupt the normal running operation (Laghari et al., 2024; ŞİMŞEK & ATILGAN, 2023). In MQTT, what these attacks do is that they seek to exhaust the broker’s resources such as CPU, RAM or Network Bandwidth or to disrupt the communication channels.

In the previous section, I mentioned that Alatram et al. (2023) documented that DoS/DDoS attacks on MQTT by creating a dataset using a physical IoT testbed with Raspberry Pi 3 Model B+ devices demonstrating that various DoS scenarios such as “CONNECT”, “PUBLISH” and TCP SYN flooding and consequently generating significant malicious MQTT traffic, having impact on the performance and availability of both the broker and connected devices. They also found out that these attacks could be effective even with limited payload sizes or QoS levels which is a fundamental vulnerability. Husnain et al. (2022) highlighted malformed MQTT packets could consume resources more causing delays in legitimate traffic on a Raspberry Pi 4 which is acting as a MQTT broker.

DoS attacks have an impact that goes beyond just disrupting services; they can deplete energy in battery-powered IoT devices and jeopardize the reliability and integrity of data transfer (Hussein & Nhlabatsi, 2022). While remedies such as limiting connections per subscriber or monitoring suspicious traffic have been proposed, these attacks continue to pose a danger due to MQTT's asynchronous nature and the resource limits of IoT devices (ŞİMŞEK & ATILGAN, 2023).

## 2.4.2 Data Tampering or Man-in-the-Middle (MitM) Attacks

MitM attacks pose a significant threat to MQTT, especially considering the protocol's default use of unencrypted communication. A MitM scenario occurs when an attacker intercepts communication between two legitimate entities, allowing them to eavesdrop, change, or insert malicious material (Karn, 2024; Laghari et al., 2024).

SIMSEK and ATILGAN (2023) demonstrated MitM attacks on MQTT with an ESP8266 NodeMCU as a sensor and a Kali Linux computer (presumably running on a Raspberry Pi or similar) as the attacker. Their investigations demonstrated that an attacker could readily acquire plaintext MQTT credentials (usernames and passwords) and message payloads using tools such as Wireshark and Ettercap. Once intercepted, the attacker might change and retransmit messages, thereby changing sensor data or control orders in real time. This is especially useful when MQTT does not use TLS/SSL encryption. Attackers might bypass even with the encryption by targeting the application layer where data is decrypted (Alsabbagh et al., 2024).

Hussein and Nhlabatsi (2022) described a MitM attack against a ZigBee/CoAP-based smart lighting system in which an attacker's Raspberry Pi 3 B+ intercepted messages between the IKEA Home Smart application (using CoAP) and the smart bulb (using ZigBee). The attacker may decode ZigBee communication (after obtaining the installation code) and then modify commands to invert bulb statuses (e.g., turn on/off) or change colors. This demonstrates the cross-protocol nature of some MitM attacks, where MQTT (or CoAP) functions as the bridge for control. Karn (2024) further demonstrated MitM proxy attacks on HTTP/HTTPS traffic, which illustrates the potential for credentials compromise and sensitive data leak even with the secure channels if the certificates are counterfeited.

The primary vulnerability exploited here is the trust between the client and broker, as well as the lack of effective end-to-end message integrity checks that are difficult to circumvent or spoof. The repercussions are serious, including compromising the IoT system confidentiality, integrity, and possibly availability (Laghari et al., 2024).

## 2.4.3 Unauthorized Access and Brute-Force Attacks

In many MQTT brokers, the default configurations often lack authentication which is a significant vulnerability enabling unauthorized access. This security absence allows any client (known or unknown) to connect, publish, or subscribe to the topics freely. It is worth noting that a significant fraction of publicly exposed MQTT servers found by Shodan allow unauthorized access. Attackers can exploit this by simply connecting to the broker and publishing malicious messages or subscribing to all topics, resulting in data breaches or dangerous device activities (Laghari et al., 2024).

To overcome the basic authentication, if it is enabled, Brute-Force attacks are a common option. It is proved that brute-force assaults against a MQTT broker using Metasploit Framework on a Kali Linux computer may successfully guess usernames and passwords (ŞİMŞEK & ATILGAN, 2023). (Karn, 2024) also described brute-forcing approaches against web apps (using Python scripts) and SSH services (using Hydra) hosted on a Raspberry Pi, emphasizing how weak or default passwords can be quickly hacked, can lead to full control over compromised systems, allowing data theft, unauthorized command executions and system disruption.

The ease with which these attacks can be carried out highlights the vital necessity for robust authentication policies and rate-limiting systems on MQTT brokers.

## 2.4.4 Information Leakage and Spoofing

The flexible topic subscription model of MQTT, with the usage of wildcard such as “#” and “+”, can be an information leakage source if it is not properly managed. By default, a client can subscribe to any topic freely potentially gaining access to the sensitive data (Roldán-Gómez et al., 2022; Laghari et al., 2024).

Polisiani et al. (2025) investigated the creation of "covert channels" by modifying MQTT topic names (for example, changing case sensitivity), proving that attackers can conceal data within ostensibly valid communication, making detection difficult.

Spoofing attacks are also a significant concern in which the attacker masquerades as a legitimate user or device.

A "Disconnect Wave" attack on MQTT-SN is defined as an attacker repeatedly connecting with a genuine device's “ClientId”, effectively ejecting the real device from the network and preventing it from rejoining. This can result in a denial of service or allow the attacker to take over the identity of the legitimate device (Roldán-Gómez et al. 2022). Hussein and Nhlabatsi (2022) also discussed masquerade attacks, in which an attacker, having obtained the ZigBee installation code, may join the network as a genuine entity and send malicious data into the system, tricking authorized users.

## 2.4.5 Data Exfiltration and Tunneling Attacks

Apart from manipulation and DoS, MQTT can also be exploited for covert communications such as tunneling, exfiltrate the sensitive data and bypass the network restrictions. Encapsulating one protocol’s traffic within another is called tunneling which makes malicious activity appear as a legitimate MQTT communication.

It is proved that a revolutionary tunneling mechanism is using MQTT. They demonstrated how HTTP/HTTPS communication may be encased in MQTT payloads and transferred between a Raspberry Pi 3 functioning as a tunnel client and a remote server. This methodology enables data exfiltration even across firewalls that accept MQTT traffic, demonstrating a stealthy form of data leakage.

It is also explored the usage of Small Language Models (SLMs) to produce realistic but malicious MQTT topics for covert communication, demonstrating that complex strategies can be employed to avoid detection by mimicking typical traffic patterns. The ability to conceal information within several MQTT packet fields, such as topic names and payload, makes these covert channels difficult to discover using typical methods (Polisiani et al., 2025).

# 2.5 Countermeasures and Ongoing Challenges

The above-mentioned topics address the various countermeasures for mitigation of MQTT exploitation, and some of the common defense strategies include:

- Strong Authentication and Encryption: Implementing TLS/SSL, mutual TLS (mTLS), or lightweight encryption algorithms (e.g., TinyJAMBU, Speck, XXTEA, Ascon, Xoodyak, as studied by Keenan, 2025) is crucial. However, the computational and memory overhead of TLS/SSL remains a significant hurdle for small devices (Keenan, 2025; Laghari et al., 2024). Keenan (2025) found that while TinyJAMBU performed well in terms of round-trip time and security features, other lightweight ciphers like Speck showed faster encryption/decryption times on ESP8266 and USB Nano devices.

- Access Control Lists (ACLs): Publisher/Subscriber permission should be restricted. This policy may not apply to all but must apply to some specific topics related to the sensitive data (Laghari et al., 2024; Alsabbagh et al., 2024).

- Intrusion Detection System (IDS): IDS should be deployed to monitor network traffic for suspicious activity, patterns, behavior, or malformed packets (Alotaibi et al., 2024; Husnain et al., 2022). AI/ML-based IDS are becoming more popular due to their ability to detect the anomalies (Alotaibi et al., 2024; Laghari et al., 2024). It is clearly demonstrated that an IDS could detect and block malformed MQTT packets which prevent CPU overload on the broker (Husnain et al., 2022).

- Packet Validation: To prevent the exploitation of protocol implementation flaws, protocol validation checks should be stricter such as including packet length, required fields or logical errors (Husnain et al., 2022).

- Firmware Updates: Finally, this is the most basic and crucial part, which is regularly patching the device firmware for addressing the known vulnerabilities (Husnain et al., 2022).

Despite these efforts, providing comprehensive security remains difficult due to the dynamic nature of threats, the resource constraints of IoT devices, and the requirement for scalable and interoperable solutions. The consensus is that a multi-layered security approach is required, integrating protocol-level improvements with network and application-level defenses (Laghari et al., 2024; Delgado et al., 2023).

# 2.6 Research Gaps and Thesis Contributions

The security community has long recognized that successful attacks frequently include numerous layers of exploitation. However, current literature rarely combines the physical and network domains in a single attack paradigm. Most MQTT research focuses on software-side vulnerabilities such broker misconfiguration, denial-of-service attacks, and authentication bypasses. Meanwhile, USB-based attacks are mostly addressed in terms of local privilege escalation or malware delivery, rather than as a means for protocol abuse.

The existing body of literature provides a strong foundation for understanding MQTT vulnerabilities and exploitation methods on various IoT hardware. Many studies successfully demonstrate attacks using Raspberry Pi models (Alatram et al., 2023; Alsabbagh et al., 2024; Hussein & Nhlabatsi, 2022; Karn, 2024; Polisiani et al., 2025; Vaccari et al., 2021) or evaluate performance on ESP8266/ESP32 devices (Keenan, 2025; Kodali & Mahesh, 2016; ŞİMŞEK & ATILGAN, 2023). However, a specific and comprehensive "Design and Demonstration of Protocol Exploitation Attacks on MQTT-Based IoT Devices Using Raspberry Pi Pico and ESP8266" as an integrated, practical system for both attack and target roles, remains a niche that this thesis aims to explore.

While ESP8266 has been effectively used as an IoT client and target for MQTT attacks (SIMSEK & ATILGAN, 2023), and Raspberry Pi Pico has been explored for its capabilities in constrained environments (Keenan, 2025) and for "Bad USB" type attacks (Karn, 2024), there is a limited explicit focus on:

- Combined Exploitation: Detailed methodologies and practical demonstrations that specifically leverage the unique characteristics of both a Raspberry Pi Pico (as a highly constrained, potentially attacker-side or target-side IoT device) and an ESP8266 (as a common IoT endpoint) within a single MQTT exploitation attack scenario. This enables analysis of inter-device communication vulnerabilities in a mixed hardware environment.

- Pico as an Attacker/Target in MQTT Exploitation: While Pico’s general capabilities are known, its specific role in designing or demonstrating MQTT protocol exploitation attacks (beyond simple data transmission or encryption benchmarking) is less detailed in the recent literature provided. This thesis can highlight how Pico’s low-cost, minimal form factor, and programmability make it a potent tool for stealthy or resource-efficient attacks, or conversely, how it behaves as a vulnerable target.

- Direct Comparison/Interaction: Research often focuses on either broad survey of vulnerabilities or specific attack types on generic hardware. This thesis will provide concrete, hands-on evidence of exploitation by focusing on the interplay between these two distinct yet common IoT microcontrollers.

It is underlined the significance of multi-vector threats in IoT, highlighting that chained attacks frequently begin with physical access and progress through lateral network movement. They argue that combining physical access with unsecure messaging technologies could enable attackers to circumvent established security measures (Rajan et al., 2023). However, their research was theoretical in nature and lacked a practical demonstration with genuine hardware.

This thesis will contribute to the understanding of IoT security by providing a practical framework and demonstration that goes beyond theoretical discussions or simulations. The proposed framework contributes by integrating USB HID payloads and MQTT message exploitations into a single proof of concept attack chain by using Raspberry Pi Pico and ESP8266 in concert, reflecting the realistic attacker behavior and provides a replicable methodology for testing the resilience of the MQTT-driven systems.

# 2.7 Critical Analysis and Future Work Orientation

This literature review justifies the proposed research which focuses on developing a practical and reproducible toolkit for simulating combination of physical and protocol layer attacks on MQTT-driven systems. The outcomes of this research are expected to benefit penetration testers, educators and IoT developers seeking to analyze, evaluate and harden the related IoT systems.

As future research, advanced detection mechanisms could be explored such as attack chains, secure MQTT extensions integration (e.g., MQTT-SN, TLS with client certificates), and broader-network IoT ecosystem testing with other protocols like CoAP and Zigbee.

# 8. Literature Review Matrix Table

| No. | Author(s) and Years | Title | Research Focus | Methodology | Key Findings | Relevance |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Alaa Alatram, Leslie F. Sikos, Mike Johnstone, Patryk Szewczyk, James Jin Kang, 2023 | DoS/DDoS-MQTT-IoT: A dataset for evaluating intrusions in IoT networks using the MQTT protocol | Development of a benchmark dataset for MQTT-based DoS/DDoS attacks in IoT systems. | Created a labeled dataset containing normal and attack traffic for MQTT over IoT. | The dataset improves the evaluation of IDS and ML-based threat detection in MQTT environments. | Provides data foundation for testing the effectiveness of MQTT-based intrusion detection systems and simulations. |
| 2. | Wael Alsabbagh, Samuel Amogbonjaye, Chaerin Kim, Peter Langendoerfer, 2024 | Pirates of the MQTT: Raiding IIoT Systems with a Rogue Client | Identification and analysis of rogue MQTT clients targeting Industrial IoT. | Real-world emulation of rogue clients interacting with open MQTT brokers. | Rogue devices can flood brokers, manipulate messages, and trigger unauthorized actions. | Validates the feasibility of offensive MQTT client behavior, useful for red-team-style testing. |
| 3. | Cristian D’Ortona, Daniele Tarchi, Carla Raffaelli, 2022 | Open-Source MQTT-Based End-to-End IoT System for Smart City Scenarios | Exploration of MQTT for building scalable, resilient IoT platforms in urban systems. | Deployment and testing of a smart city prototype using MQTT. | MQTT was effective for lightweight smart city communications but required strong endpoint security. | Establishes protocol relevance in critical systems and highlights need for security in MQTT-based infrastructures. |
| 4. | Dan Dinculeană, Xiaochun Cheng, 2019 | Vulnerabilities and Limitations of MQTT Protocol Used between IoT Devices | Comprehensive analysis of known MQTT protocol vulnerabilities. | Systematic literature review and categorization. | Identified 8 core vulnerability categories, including session hijacking and lack of encryption. | Establishes a theoretical framework for MQTT threat modeling. |
| 5. | Antonio Francesco Gentile, Davide Macrì, Domenico Luca Carnì, Emilio Greco, Francesco Lamonaca, 2024 | A Network Performance Analysis of MQTT Security Protocols with Constrained Hardware in the Dark Net for DMS | Evaluates security adaptations of MQTT in restricted computing environments such as darknet or embedded devices. | Performance evaluation of TLS, DTLS, and custom encryption mechanisms. | Trade-off between cryptographic strength and latency/resource use must be balanced for secure MQTT. | Reinforces constraints of embedded MQTT security for devices like ESP8266. |
| 6. | HiveMQ, 2025 | MQTT Essentials: The Ultimate Guide to MQTT for Beginners and Experts | Provides a full protocol-level explanation of MQTT operations | Educational series complied by MQTT vendor experts. | Convers broker roles, QoS levels, retained messages, and clean session behavior. | Supports protocol literacy for designing offensive and defensive MQTT experiments. |
| 7. | Muhammad Husnain, Khizar Hayat, Enrico Cambiaso, Ubaid U. Fayyaz, Maurizio Mongelli, Habiba Akram, Syed Ghazanfar Abbas and Ghalib A. Shah, 2022 | Preventing MQTT Vulnerabilities Using IoT-Enabled Intrusion Detection System | Proposal of a lightweight IDS model for MQTT-based smart systems. | Designed and validated DL-based IDS for real-time detection of MQTT threats. | Model achieved high accuracy detecting message anomalies and replay attacks. | Demonstrates the potential of integrating IDS into MQTT networks and testing it using simulated attacks. |
| 8. | Noon Hussein and Armstrong Nhlabatsi, 2022 | Living in the Dark: MQTT-Based Exploitation of IoT Security Vulnerabilities in ZigBee Networks for Smart Lighting Control | Case study of insecure MQTT-based home lighting systems. | Exploit demonstration and system simulation. | Proved that weak MQTT topics and broker exposure could enable unauthorized lighting control. | Demonstrates real-world exploit potential in MQTT deployments. |
| 9. | Sameer Karn, 2024. | IT/IoT Cyber Exploration | Bachelor thesis done with the overview of cybersecurity education topics including MQTT and IoT basics. | Literature systhesis and academic focus model. | Serves as a teaching aid. | Offers educational framing, but not so useful in experimental cases. |
| 10. | John Joseph Keenan, 2025 | An Analysis Of The Encryption Algorithms Proposed For The MQTT Protocol On Resource Constrained IoT Devices | Comparison of encryption options for MQTT communications on low-power devices. | Benchmarked several TLS and symmetric encryption models. | Lightweight ciphers like “ChaCha20” ofer balance of speed and security. | Directly relevant to protocol-level security in constrained MQTT nodes. |
| 11. | Ravi Kishore Kodali, Kopulwar Shishir Mahesh, 2016 | A low cost implementation of MQTT using ESP8266 | Design of a low-cost, reliable MQTT communication model for home automation. | Prototype built using ESP8266 and NodeMCU. | MQTT works efficiently on low-cost Wi-Fi hardware with minimal latency. | Foundational for proving ESP8266 can host reliable MQTT operations. |
| 12. | Hassan Kurdi, Vijey Thayananthan, 2021 | Authentication mechanisms for IoT system based on distributed MQTT brokers: review and challenges | Survey of broker authentication methods and their effectiveness. | Comparative review of static vs dynamic authentication systems. | Static credentials are common but weak in general; tokenized auth improves security. | Underscores need for credential management in MQTT broker setup. |
| 13. | Shams UI Arfeen Laghari, Wenhao Li, Selvakumar Manickam, Priyadarsi Nanda, Ayman Khallel Al-Ani, Shankar, 2024 | Securing MQTT Ecosystem: Exploring Vulnerabilities, Mitigations, and Future Trajectories | Broad threat analysis and mitigation strategies across MQTT stack. | Security lifecycle review and tool-based evaluation. | Proposed a multilayered defense model combining TLS, ACL, and anomaly detection. | Maps out holistic defenses which are useful in simulation environments. |
| 14. | Camilla Cespi Polisiani, Maria Carla Calzarossa, Marco Zuppelli, Luca Caviglione, and Massimo Guarascio, 2025 | Improving MQTT Security Through the Generation of Malicious Test Cases | Introduced a test suite for evaluation of MQTT broker behavior under attack. | Generated and ran structured malicious payloads. | Brokers show inconsistent handling of malformed packets and with fuzzing, it uncovers crashes. | Enables building a fuzzing module for protocol exploitation. |
| 15. | Meenaxi M Raikar, Meena S M, 2021 | Vulnerability assessment of MQTT protocol in Internet of Things (IoT) | Real-world assessment of MQTT brokers’ response to crafted input. | Penetration testing across open MQTT brokers. | Found widespread issues with unauthenticated access and topic spoofing. | Reinforces practical valud of MQTT-based protocol attacks. |
| 16. | José Roldán-Gómez, Javier Carrillo-Mondéjar, Juan Manuel Castelo Gómez and Sergio Ruiz-Villafranca, 2022 | Security Analysis of the MQTT-SN Protocol for the Internet of Things | Evaluation of MQTT-SN for sensor-based networks. | Protocol behavior review and comparative simulation. | MQTT-SN lacks standard security features such as spoofing and replay. | Show limits of variants used in low-power networks. |
| 17. | Mustafa Muhammed Şimşek , Emrah Atılgan, 2023 | Attacks on Availability of IoT Middleware Protocols: A Case Study on MQTT | Accesses susceptibility of MQTT to availability-based attacks. | Case study including analysis on impact. | MQTT systems are highly vulnerable to QoS abuse and broker saturation. | Confirms protocol’s DoS exposure critical for attack emulation. |